# HOW TO BACKUP MY ALLSTAR NODE’S SD CARD

# WHAT DOES THE SD CARD IN MY NODE DO? - AllStar Nodes typically run on small single-board computers, like a Raspberry Pi. That computer boots entirely from a microSD card, which contains everything the node computer needs to function:

- The computer’s operating system.

- Asterisk, Hamvoip and Supermon software.

- Your node’s configuration settings.

- Custom scripts, audio files, and tuning data.

- Network settings and passwords.

### CORRUPTED SD CARDS - There are many causes of corrupted SD cards. Some can be prevented. Others occur randomly, or by operator error.

- Accidental misconfiguration during user reconfiguration or experimentation.

- Power failures, low supply voltages, and power line noise.

- File system and cell write errors.

- RF interference or noise.

### WORN OUT SD CARDS – All SD cards have a maximum number of reliable write cycles, and limited data retention time.

- Typically, 10,000 write cycles for a budget SD card to 100,000 write cycles for a premium SD card.

- Nonvolatile node data is repeatedly saved to the SD card. System logs, database files, state files, and Pi swap files may be frequently written to the SD card.

- Some manufacturers also specify that their SD cards will only reliably retain data for a maximum of 10 years.

### HOW DOES THE BACKUP SD CARD HELP US? - A backup SD card lets you instantly restore your node’s operating system and custom settings, instead of having to rebuild it from scratch. By replacing the original SD card with the backup SD card, your node’s previous settings will be retained. Those include custom audio levels, COS/CTSS settings, and control scripts and telemetry. After swapping out the SD cards, the node should operate exactly as it did with the original SD card. It is a plug and play repair.

### NOTE: A node’s SD card contains the majority of the nodes configuration settings. Many nodes have a PiHat radio receiver, like the SA-818. This receiver is programmed with several parameters that set up the nodes radios. If there is an SD card failure, the receiver seldom needs to be reprogrammed.

HOW DO I BACKUP MY SD CARD

DOWNLOAD THE SD CARD FORMATTER AND SD CARD IMAGE CREATOR.

Two programs will be needed to backup your SD card. The first is an SD card format program. It completely erases any existing data on an SD card. The second is a program that copies an image from the original SD card into the backup SD card.

- Download and install SD Card Formatter from SD Associates.

SD Memory Card Formatter for Windows Download - SD Association

- Download and install Win32 Disk Imager from SourceForge.net.

Win32 Disk Imager download | SourceForge.net

FORMAT THE SD CARD THAT WILL BE USED FOR THE BACKUP.

- Run the SD Card Formatter Program.

- The window shown in Figure 1 will be displayed.

Figure 1

- Insert the backup SD Card into your computers SD card socket.

- In the Select Card prompt, type in the location of the card that needs to be formatted. A drop down arrow is also be shown at the right of the entry prompt. If the card is shown on the drop down, you may select it there.

- Under Formatting Options, click on the Quick Format circle to reformat the disk file system. If you want to completely erase all data on the disk, choose OVERWRITE FORMAT. This will take a long time to complete, and is seldom necessary.

- Delete any names under the Volume Label prompt.

- Click on the FORMAT button.

- A warning prompt will be given. It indicates that everything on the SD card will be erased. Click on the YES button.

- A green progress bar will show the formatting status.

- A successful formatting prompt will be given after the SD card is formatted. Click on OK.

- Exit the SD Card Formatter program.

SAVE A DISK IMAGE FROM YOUR ORIGINAL SD CARD TO A BACKUP SD CARD.

- Run the Win32 Disk Imager Program. The window shown in Figure 2 will be displayed.

- Insert the SD Card that you want to backup into your computers SD card socket.

Figure 2

- Under the Image File prompt, type in a file location and name where the Allstar Disk Image File will be kept on your computer. This file should be retained, because it provides a backup that may be used to create another copy of the SD card image. It is advisable to include the node number and date in the saved image backup. For example, try F:/123456_012326.img.

- Under the Device prompt, select the backup SD Disk location.

- Check Read Only Allocated Partitions.

- Click on the Read button.

- A confirm Overwrite prompt will be opened. Click on YES. A copy of the original Allstar disk image will be saved to your computer. A green status bar will show the file transfer progress. This may take a while.

- When the transfer is complete, a Read Successful message will be displayed. Click on the OK Button.

- Remove the original SD card, and insert the backup SD card that was formatted above.

- The previously entered Image File location will still be visible in the prompt window. If not, reenter it.

- The Device should be set to where the backup SD card is located.

- Click on the WRITE button.

- A Confirm overwrite prompt will be displayed. Click on the YES button.

- The write progress will be shown on the green status bar.

- After the write is complete, a successful write message will be displayed. Click the OK button.

- Exit this program, and remove the backup SD card.

- The newly created backup SD card is now ready for use.

HELPFUL HINTS.

- It’s often difficult to insert the SD card into the card socket on a Pi. To make it easier, attach a folded over piece of tape to the end of the SD card. This will allow you to grab the SD card by the newly applied flap.

- It is easy to misplace your backup SD card, due to its small size. Try taping the backup SD card inside the node, or to the bottom of it.

- Too often Hams misplace the node registration data. Fill out the table listed below, and tape it to the bottom of your node. It’s small enough to fit on the back of most nodes.

Allstar Node: xxxxxxxxxx

Password: xxxxxxxxxx

Root Login Password: xxxxxxxxxx

Master Password: xxxxxxxxxx

IP: xxx.xxx.xxx.xxx

Mask: xxx.xxx.xxx.xxx

Gateway: xxx.xxx.xxx.xxx

IAX Port: xxxx

Allmon Password: xxxxxxxx

Supermon User Name: xxxxxxxxxx

Supermon User Password: xxxxxxxxxx

W2RBC  (Ver 1.3)